HW Test 9: Interrupts-General

When the X-bit of the CCR is [a], the non-maskable interrupt is enabled.

Zero

Which of the following are potential sources for maskable interrupts?

Port H, Port J, PWM, SCI, SPI, Port P

The _____ register is used to "boost" the priority of an interrupt source.

HPRIO

_______ interrupts can occur in three ways: an interrupt request on the _______ pin, an unimplemented software instruction trap, or a software interrupt instruction, _______.

non-maskable, XIRQ, SWI

An interrupt vector is _______ bits in the 9S12.

16

______ interrupts can be disabled and enabled through software.

Maskable

Interrupts can be divided into two types: ____ which can happen at any time, and ____ which occur at predetermined intervals.

Asynchronous | Synchronous

Write a single C instruction using a bitwise operator that toggles pin 1 on Port H without affecting the value of any other pin.

PTH = PTH ^ 0x02;

To have the processor run runit() when timer channel five produces an interrupt we would run the following instruction: SETVECTOR([a], [b]);

[a] = 0xFFE4 | [b] = runit

The [a] bit in the CCR controls the non-maskable interrupt of the 9S12.

x

Write a C instruction to reset the interrupt flag for pin 1 of Port H.

PIFH = PIFH | 0x02;

When the X-bit of the CCR is , the non-maskable interrupt is disabled.

1

If programmed to, a[n] [a] reset can occur when a processor has entered an infinite loop.

COP/Computer operating properly/Watchdog

An interrupt vector is [a] bits in the 9S12.

16

Which of the following are potential sources of maskable interrupts?

PWM, Port J, SCI, Port H, SPI, Port P

What does SETVECTOR(0xFFCC, my_isr) do?

Tells processor to put address of my_isr at address 0xFFCC

A _______ occurs when a low-active reset pin is activated at startup to guarantee a known initial state.

Power-On Reset

A _____ is an exception with a “one-way ticket.”

Reset

To have the processor run doit() when pin 1 of Port H goes low, we would run the following instruction:

SETVECTOR([a], [b]);

SETVECTOR(0xFFCC, doit);

Write a single C instruction to reset only the interrupt flags on Port H which are high.

PIFH = 0xFF;

To have the processor run doatod() when timer the A/D 1 produces an interrupt we would run the following instruction: SETVECTOR([a], [b]);

SETVECTOR(0x,FFCC, doatod)

To have the processor boost the priority of an interrupt on pin 5 of Port H, we would put a value of [a] in the [b] register.

a = 0xCC	b = HPRIO

The [a] bit of the CCR controls the maskable interrupts of the 9S12

I

There are two types of interrupts which are determined by their timing. [a] interrupts can be used to switch between processes at pre-determined intervals.

Synchronous

There are two types of interrupts which are determined by their timing. An example of  a/an [a] interrupt is a timer used to write to a display at specified periodic intervals.

Synchronous

There are two types of exceptions: [a] e.g. and external interrupt, and [b], e.g. a divide by zero.

Hardware, Software

See above Table (table in previous question). To have the processor boost the priority of an interrupt on timer 7, we would put a value a [a] in the [b] register.

0xE0, HPRIO

A[n] [a] reset occurs when the processor detects the system clock is not within a valid frequency range.

Clock Monitor

[a] exceptions can be caused by illegal or erroneous instructions.

Software

When the X-bit of the CCR is [a], the non-maskable interrupt is enabled.

0

To have the processor run my_isr() when pin 3 of Port H goes low, we would run the following instruction: SETVECTOR([a], [b]);

SETVECTOR(0xFFCC, my_isr);

If programmed to, a[n] [a] reset can occur when a processor has entered a brown-out (low-voltage) condition.

COP/Computer operating property/ Watchdog

[a] interrupts can only be disabled through internal processor hardware.

Non-maskable

An [a] is a break in normal program flow and comes in two types: interrupts and resets.

Exception

The X-bit of the CCR cannot be set to [a] through software.

1

When the I-bit of the CCR is [a], all maskable interrupts are potentially enabled.

0

A[n] reset occurs when a low-active pin is activated by a user pressing a switch to force the processor to reset.

External

When the I-bit of the CCR is [a], all maskable interrupts are disabled.

1

Write a single C instruction using a bitwise operator that toggles pin 1 on Port H without affecting the value of any other pin.

PTH = =PTH ^ 0x02;

Write a single C instruction to reset the interrupt flags for every pin on Port H.

PIFH = 0xFF;

HW Test 10: RTI General

If the ECLK frequency is 7.6 MHz, what is the Core Clock frequency?

Core_Clock_Freq = 2*ECLK_Freq

15.2

If the Core Clock frequency is 11.6 MHz, what is the ECLK frequency?

ECLK_Freq = Core_Clock_Freq/2

5.8

What bit in the 9S12 turns on or off the RTI?

RTIE

What bit in the 9S12 tells whether or not the RTI has expired?

RTIF

What clock is the RTI frequency directly based off?

Oscillator Clock

Given the following register settings in the 9S12

RTICTL = 0x00

CRGINT = 0x92

CRGFLG = 0x8A

and a Core clock frequency of 10 MHz

what is the RTI frequency in Hz?

RTICTL determines interrupt frequency, so at 0x00, interrupts are disabled

0

HW Test 10: RTI - What is Counter Value

RTI Divisor = 1024*(RTICTL[lower in decimal]+1)*2^(RTICTL[upper in decimal]-1)

RTI Rate = CCF/RTI Divisor

Seconds = Counter / RTI Rate

RTI Divisor = 1024*(RTICTL[lower in decimal]+1)*2^(RTICTL[upper in decimal]-1)

RTI Rate = CCF/RTI Divisor

Counter = Seconds * RTI Rate

(Always round to nearest integer)

HW Test 10: RTI-What is Frequency

RTI Divisor = 1024 * (RTICTL(lower in decimal) +1) * 2^(RTICTK(upper in decimal)-1)

RTI Frequency = Core Clock Frequency (MHz) / RTI Divisor

RTI Divisor = 1024 * (RTICTL[lower in decimal] + 1) *2^(RTICTL[upper in decimal] - 1)

Period (ms) = RTI Divisor / Clock Freq (MHz)

HW Test 10: RTI-What is RTICTL

Table # = Core Frequency * Seconds

Find upper and lower bits by plugging number into table

Table # = Core Frequency / Hz

Find upper and lower bits by plugging number into table

HW Test 11: Timers General

Which Timer Channel can affect all other Timer Channels' behavior.

Channel 7

What 9S12 integrated functionality counts clock pulses until an input signal changes state?

Input Capture

What 9S12 integrated functionality works like a “stop watch?”

Input Capture

What 9S12 integrated functionality works like a "Alarm Clock?"

Output Compare

What does the following line of C instruct the compiler to do? Why/when is it necessary?

#define _LP(x) *(unsigned int *)(x)

It replaces _LP(x) with *(unsigned int *)(x) where x is a variable name or constant. This statement is used with 16 bit operations.

The diagram below shows the hardware in a 9S12 used to produce a[n] [a] [b].

Output Compare

Input Capture

In the diagram below, what signal/bit corresponds to the letter H? I?

(See Picture at Beginning)

What signal values/transitions can the output compare system produce at a pin?

Toggle, Low Value, High Value

How many input capture channels are there on the 9S12?

8

How many Output Compare channels are there on the S12?

8

What signal values/transitions can input capture be initialized to detect?

Rising Edge and Falling Edge

When does the TOF bit get set?

When the free-running counter goes from 0xFFFF to 0.

What is the lowest value possible in the TCNT register?

0, zero, 0x0000

What is the highest value possible in the TCNT register?

0xFFFF

In the diagram below, what signal/bit corresponds to the letter B? G?

(SEE Picture at Beginning)

What signal values/transitions can the output compare system produce at a pin?

Toggle, Low Value, High Value

Which of the following registers are 16 bits?

TC4, TC5, TCNT

HW Test 11: Frequency

PR = 2^(TSCR2(first 3 bits of lower[binary]))

Bus Clock = Core Clock / 2

Frequency = Bus Clock / PR

HW Test 11: Period

PR = 2^(TSCR2(first 3 bits of lower[binary]))

Bus Clock = Core Clock / 2

Frequency = Bus Clock / PR

Clock_Period = 1/Frequency = PR / Bus Clock

HW Test 11: Clock for Period or Frequency

PR = 2^(TSCR2(first 3 bits of lower[binary]))

Bus Clock = PR / Clock_Period

CCF = Clock_Freq*2*2^(TSCR2(first 3 bits of lower[binary]))

HW Test 11: Prescale

Match with closest Prescale Factor

Bus Clock = Core Clock / 2

PR = Bus Clock*Clock_Period

HW Test 11: Prescale and Count

(2^PR)(COUNT) = (BusClk*Period)/2

*pick value of PR to get whole # for Count

HW Test 11: Set Edge Detection

*TCTL3 [7:4] *TCTL4[3:0]

Compare with table

Make each pin a two digit Binary Number using B and A

Convert to Hex

Compare with table

Make each pin a two digit Binary Number using B and A

Convert to Hex

HW Test 11: What Edge Detection

Pin 7 - Bit 7 and 6 of TCTL3

Pin 6 - Bit 5 and 4 of TCTL3

Pin 5 - Bit 3 and 2 of TCTL3

Pin 4 - Bit 0 and 1 of TCTL3

Pin 3 - Bit 7 and 6 of TCTL4

Pin 2 - Bit 5 and 4 of TCTL4

Pin 1 - Bit 3 and 2 of TCTL4

Pin 0 - Bit 1 and 0 of TCTL4

Use Directions and Table above

HW Test 11: TCNT Calc

Bus_Clock = Core_Clock / 2

PR = 2^(TSCR2(first 3 bits of lower[binary]))

TCNT = ((Bus_Clock*Time) / PR) + Inititial_TCNT

HW Test 11: TCNT Max Count Calc

Bus_Clock = Core_Clock / 2

PR = log2((Bus_Freq * Time)/65536 = 4.596

PR = 2^(TSCR2(first 3 bits of lower[binary]))

Bus_Clock = (65536*PR)/Time(sec)

Bus_Clock = Core_Clock / 2

PR = 2^(TSCR2(first 3 bits of lower[binary]))

Time(sec) = (65536*PR)/Bus_Clock

HW Test 11: Overflow Calc

PR = 2^(TSCR2(first 3 bits of lower[binary])

Time(sec) = (65536 - Initial_Count) * PR / Bus_Clock

HW Test 11: Set Output Compare Levels

*TCTL1 [7:4] *TCTL2 [3:0]

Compare with table

Make each pin a two digit Binary Number using B and A

Convert to Hex

Compare with table

Make each pin a two digit Binary Number using B and A

Convert to Hex

HW Test 11: What Output Compare Levels

Pin 7 - Bit 7 and 6 of TCTL1

Pin 6 - Bit 5 and 4 of TCTL1

Pin 5 - Bit 3 and 2 of TCTL1

Pin 4 - Bit 0 and 1 of TCTL1

Pin 3 - Bit 7 and 6 of TCTL2

Pin 2 - Bit 5 and 4 of TCTL2

Pin 1 - Bit 3 and 2 of TCTL2

Pin 0 - Bit 1 and 0 of TCTL2

Use Directions and Table Above

HW Test 11: Square Wave Calc

Bus_Clock = Core_Clock / 2

PR = 2^(TSCR2(first 3 bits of lower[binary]) + 1)

Count = (Bus_Clock * Time(sec)) / PR

Bus_Clock = Core_Clock / 2

PR = 2^(TSCR2(first 3 bits of lower[binary]) + 1)

Time(sec) = (Count * PR) / Bus_Clock

HW Test 11: Square Wave Calc with Overflow

Bus_Clock = Core_Clock / 2

PR = 2^(TSCR2(first 3 bits of lower[binary]) + 1)

Wave_period = (Count * Offset * PR) / Bus_Clock

HW Test 11: Application - Shaft - Max RPMs

PR = 2^(TSCR2(first 3 bits of lower[binary]))

If minimum, time = (65536 * PR) / Bus_Freq

If maximum, time = PR / Bus_Freq

RPMs = 60 / (time * #magnets)

HW Test 13: CFORC

Put a 1 where your channel is, everything else is 0. Convert to Hex

HW Test 13: Delay Count

Using Second Line of Code, Get bit 1 and bit 0 and match with table

Match number of cycles with table, that become bit 1 and bit 0

DLYCT =0x0#

HW Test 13: DLYCT Calc

Bus_Clock = Core_Clock / 2

Count_Value = Bus_Clock (Hz) * Time(sec)

Count_Value < 256 => 1

Count_Value < 512 => 2

Count_Value < 1024 => 3

Count_Value > 1024 => 0

HW Test 13: OC7M-OC7D

Write out bits - ???? ????

If bit not mentioned, replace with 0

If bit is sets to 1, replace with 1

For bits that clear to 0: OC7M will be 1, OC7D will be 0

If a bit is 1 in OC7M, then the bit from the OC7D overrides the bit in PTT, otherwise bit is same as PTT

HW Test 14: Find PWM Clock Freq

1. Bus_Clock = Core_Clock / 2

2. To determine whether it will be A/SA, or B/SB:

A/SA if Channel 0,1,4,5

B/SB if Channel 2,3,6,7

3. For PR:

If A/SA: PR = 2^(PWMPRCLK(bits[2:0]))

If B/SB: PR = 2^(PWMPRCLK(bits[6:4]))

4. Clock A/B = Bus_Freq / PR

5. To determine whether S clock or not:

If  the corresponding bit of your channel and PWMCLK is a 1, then it will be an S clock

Otherwise, the number you got in part 4 is your answer

6. If SA/SB clock

Sa/SB = (Clock A/B (Part 4))/ (2*(Decimal value of either PWMSCLA/PWMSCLB))

*DO NOT ROUND * MAKE SURE ITS kHz

HW Test 14: Find PWMPER

1. Bus_Clock = Core_Clock / 2

2. To determine whether it will be A/SA, or B/SB:

A/SA if Channel 0,1,4,5

B/SB if Channel 2,3,6,7

3. For PR:

If A/SA: PR = 2^(PWMPRCLK(bits[2:0]))

If B/SB: PR = 2^(PWMPRCLK(bits[6:4]))

4. Clock A/B = Bus_Clock / PR

5. To determine whether S clock or not:

If  the corresponding bit of your channel in PWMCLK is a 1, then it will be an S clock

Otherwise, the number you got in part 4 is your answer (Skip to 7)

6. If SA/SB clock

Sa/SB = (Clock A/B (Part 4))/ (2*(Decimal value of either PWMSCLA/PWMSCLB))

7. Period_One_Click = 1 / (Answer from part 4 or 6)

8. To Find PWMPER

If the corresponding bit of your channel in PWMCAE is a 0

PWMPER = (PWM Period(given)) / Period_One_Click

If the corresponding bit of your channel in PWMCAE is a 1

PWMPER = (PWM Period(given)) / (2 * Period_One_Click)

9. To Find PWMDTY

If the corresponding bit of your channel in PWMPOL is a 0

PWMDTY = PWMPER * (1-(DutyFactor(in %)/100))

If the corresponding bit of your channel in PWMPOL is a 1

PWMDTY = DutyFactor(in %) * (PWMPER / 100)

*ROUND TO NEAREST WHOLE NUMBER

HW Test 14: Find PWM PERIOD

1. Bus_Clock = Core_Clock / 2

2. To determine whether it will be A/SA, or B/SB:

A/SA if Channel 0,1,4,5

B/SB if Channel 2,3,6,7

3. For PR:

If A/SA: PR = 2^(PWMPRCLK(bits[2:0]))

If B/SB: PR = 2^(PWMPRCLK(bits[6:4]))

4. Clock A/B = Bus_Clock / PR

5. To determine whether S clock or not:

If  the corresponding bit of your channel in PWMCLK is a 1, then it will be an S clock

Otherwise, the number you got in part 4 is your answer (Skip to 7)

6. If SA/SB clock

Sa/SB = (Clock A/B (Part 4))/ (2*(Decimal value of either PWMSCLA/PWMSCLB))

7. Period_One_Click = 1 / (Answer from part 4 or 6)

8. To Find PWM Period

If the corresponding bit of your channel in PWMCAE is a 0

PWM Period = PWMPER(Decimal) * Period_One_Click

If the corresponding bit of your channel in PWMCAE is a 1

PWM Period = 2 * Period_One_Click * PWMPER(Decimal)

9. To Find PWM Duty

If the corresponding bit of your channel in PWMPOL is a 0

PWM Duty(%) = 100 * (PWMPER(Dec) - PWMDTY(Dec)) / PWMPER(Dec)

If the corresponding bit of your channel in PWMPOL is a 1

PWM Duty(%) = 100 * (PWMDTY(Dec) / PWMPER(Dec))

10. To Find PWM Time High

PWM Time High = PWM Period * PWM Duty

HW Test 14: Find PWM-CON

HW Test 14: Find PWMCTRL

Give a single C instruction which will stop the PWM input clock to the pre-scaler to save power in the 9S12 WAIT mode. (Leave exactly one space between each variable/symbol.)

PWMCTL = 0x08

Give a single C instruction which will stop all PWM counter in "freeze mode."

PWMCTL = 0x04

Give a single C instruction which will create a 16-bit PWM channel on pin 1 of Port P.

PWMCTL = 0x10

Give a single C instruction which will create a 16-bit PWM channel on pin 3 of Port P. (Leave exactly one space between each variable/symbol.)

PWMCTL = 0x20

Give a single C instruction which will create a 16-bit PWM channel on pin 5 of Port P.

PWMCTL = 0x40

Give a single C instruction which will create a 16-bit PWM channel on pin 7 of Port P.

PWMCTL = 0x80

HW Test 14: What PWM Form

PWMPOL to see if HIGH or LOW

LOW if corresponding Pin is 0

HIGH if corresponding Pin is 1

PWMCAE to see alignment

Left Aligned if corresponding Pin is 0

Center Aligned if corresponding Pin is 1

PWMPER in Decimal is total Period

PWMDTY in Decimal is the time it is active in that period

If PWMDTY is close to PWMPER, then active for long time

So if active for long time, if starts low, find one that stays low the most. If starts high, find one that stays high most of the time

HW Test 14: What PWM Scale

Bus_Freq = Core_Freq / 2

(Bus_Freq / fPWM(Hz)) / (2 * PWMPER) = PWMSCLA * (2^PCKA)

Plug in for left side of equations.

To solve for right side:

If Corresponding bit of PWMCAE is 1, divide entire left side of equation by 2

Make into whole number

Divide left side of equation by 2^(Highest value of something that will still give whole number) = PWMSCLA

The value that you raise (2^?) = PCKA

For the second set of numbers you need to find, SCLA can’t be above 255. If the whole number found previously is below 255: PCKA = 0; PWMSCLA = Whole number

Otherwise find the next value that will give a PWMSCLA below 255

(Alternate Way to solve in calculator)

Bus_Freq = Core_Freq / 2

Y = (Bus_Freq / fPWM(Hz)) / (2 * PWMPER)

If Corresponding bit of PWMCAE is 1, Y by 2

Calc: Solve(y=(2^?)*x,x)

Starting at 0, raise the power up by 1. The first power that gives an x value below 255 is your first PCKA, and the x value is PWMSCLA.

Then continue to raise the power by one until your x value becomes a fraction, the power and x value before the fraction is your answer for the second set