Serial Peripheral Interface (SPI) - Full Duplex

Inter Integrated Circuits (I^2C) - Half Duplex

MOSI – Master Out Slave In (Data out)

MISO – Master In Slave Out (Data in)

SCK – Serial clock

SS# - Slave Select

PIC32 has 2 SPI peripherals available

- SPIxBRG<0:8> - Baud rate register

# PIC32 has 3 comparators

# CVRCON

D to A:

Zero Error – error when input = 0

Full-Scale Error - error when input is the maximum

Relative Accuracy Error – error between actual and the line from output_max and output_min.

Monotonicity - output always increasing as input increases

Settling Time - The required for the output to reach with a D/2 of the stable output when the input changes from 0 to 2N-1, or 2N-1 to 0.

A-to-D:

Offset Error - Error of analog transition point nearest 0 Volts.

Full Scale (Gain) Error - Error of analog transition point farthest from 0 Volts.

Differential Nonlinearity Error - The maximum difference between a step D and the constant ideal D.

DNLE = Maximum |Dk – D|

A Successive Approximation Register (SAR) is an ADC where each output bit is determined sequentially, starting with the most significant bit (MSB) working down to the least significant bit (LSB) in a binary search algorithm.

Dual-Ramp ADCs use an RC Operational Amplifier Circuit coupled with a timer to measure the value of an input.

- Slower but more precise because it needs to charge and discharge a capacitor.

PIC32 A-to-D:

- 10 bit, up to 16 analog inputs, internal VREF, multiple channel scan, 16-word buffer, 8 conversion formats and can run during idle/sleep mode.

- Signed voltages: Vad = ( # / 1024 ) * VREF+

  - (VREF+ - VREF-) / 2 + Vad

  - For fractions add 1 if POSITIVE

UARTS:

Overrun Error – A new character has arrived before previous character has been read.

Underrun Condition – The UART’s transmit buffer is empty while communication has been taking place.

Framing Error – Start and/or stop bits are in error.  That is, the line is not low where a start bit should be, or it is not high where the stop bit(s) should be located.

Parity Error – The parity bit (if present) is not the value should be for the transmitted data bits.

Break Condition – A certain number of consecutive 0’s (generally more than a complete character) have been received.  Can be sent intentionally to tell the receiver communication is finished.

Baud Rate: Symbols per second

If BRGH = 1 then replace 16 with 4 ^.

Stepper Motors:

- Unlike many motors, they have full torque at stand-still and have excellent response to starting, stopping, and reversing.

- They don’t need brushes for commutation, so they have a longer motor life depending primarily on bearing and coil life.

- They can achieve very low speeds and only require open-loop control

-  Furthermore, their speed is determined by the period of these pulses, not by their voltage.

- Disadvantages:
Resonance can be a problem if not controlled.

Do not perform as well for high-speed applications.

Stepping Modes:

Wave Drive Mode: one by one A – B – A’ – B’

Full Step Drive: 2 at a time AB – BA’ – A’B’ – B’A

Half Step Drive: combo of above two AB-A-BA’-A’-A’B’-B’-B’A-A

Micro Stepping: smaller step sizes

Stator – Stationary (generally) part of the motor held in place by the outer casing of the motor.

Rotor – Inner part that rotates in response to changes in the activation pattern.  It is supported at each end by bearings and includes a projecting shaft for the connection of the load.

Stator Teeth – Radial projections on the stator which provide a more precise path for the magnetic flux to flow between the stator and the rotor and produce the motor’s movement.

Rotor Teeth – Radial projections on the rotor which are also used to provide a path for the magnetic field that produces the motor’s movement.

Multi-Stack Variable Reluctance: rotor teeth lose to (not equal) stator is multiple of phases.

12 = rotor teeth

Hybrid: Rotor teeth > Stator Teeth (still multiple of poles)